# Caravan Sites and Control of Development Act 1960

Additional Conditions for Sites with Winter Occupancy Opening

1.  	Water Supply

Water pipes must be laid to at least a depth of 0.76 meters below the ground level as recommended by United Utilities.

From this depth up to ground level and continuing up to the caravan the water pipes must be suitably insulated, to allow the system to operate effectively under adverse weather conditions.

If the above work has not been carried out then communal toilet blocks must be provided. Measures should be taken to ensure that the toilet block is provided with adequate heating and ensure that the water pipes are adequately insulated, to enable the facilities to be used outside the dates of 1 March to 15 November in any year.

Facilities should be provided on the following scales:

-   Men: 1 WC and 1 urinal per 15 caravans

-   Women: 2 WCs per 15 caravans

-   1 wash hand basin for each WC or group of WCs

-   1 shower or bath (with hot and cold water) for each sex per 20 caravans

2.  	Fire Fighting Equipment

(1)   	Frost Protection for Fire Extinguishers

All water type, foam or AFFF foam extinguishers shall be filled with an anti-freeze solution mixed in with their contents, in the concentrations and quantities stipulated by a competent person (supplier) or as stated by the extinguisher manufacturer.

The use of substances which will increase burning rates in the event of fire is prohibited and the solution shall be added by properly decanting the contents from the extinguisher and not merely by topping up.

A suitable warning label shall be placed in a prominent position on each extinguisher stating that the extinguisher has been "winterised" and specifying the particular anti-freeze solution added.

The extinguishers treated shall be so positioned that they comply with general licence conditions relating to maximum distances from caravans.

(a)   	Each winterised extinguisher shall be totally discharged at least once in every five year period in accordance with British Standard 5306 Part II.

(b)   	Records of the treatment of extinguishers, maintenance and discharge dates shall be kept by the licence holder and available for inspection on the request of an Authorised Officer.

(2)   	Frost Protection for Hose Reels

All hose reels shall be under-run and drained back to the valve. Pipework below the valve shall have an adequate level of frost protection such as to allow the system to operate effectively under adverse weather conditions.

(a)   	In the event of a site relying totally on hose reels for fire precaution, a number of winterised fire extinguishers shall also be placed in conveniently accessible locations on site, not at a greater distance from any caravan than the maximum stated in the standard licence conditions.

3.  	Roads, Gateways and Footpaths

An adequate supply of grit should be maintained to treat roads and pathways.

4.  	Letting Units

Caravan units for hire by the site owner must be provided with an adequate water supply suitably protected from frost damage. Additional heating/insulation should be provided within the unit if necessary.

The above additional winter occupancy conditions apply to holiday caravans sites, or areas of such a site, which have appropriate planning permission.

NB:	Please note where guidance or certain British Standards have been mentioned these may have been updated since the conditions were adopted.